Experiment 7.2

Fourier Series of Half-Wave Rectifier Output

Aim:

To compute and analyze the Fourier series representation of the output of a half-wave rectifier. The Aim is to understand how the Fourier series can decompose a non-sinusoidal periodic signal into its constituent sinusoidal components (harmonics), and to observe how these harmonics contribute to the overall signal.

Program:

% Fourier Series of Half-Wave Rectifier Output

clc;

clear;

close all;

% Time axis

t = 0:0.001:2*pi;

% Half-wave rectified signal

x = sin(t);

x(x < 0) = 0;

% Fourier Series approximation

N = 10; % Number of harmonics

a0 = (1/pi)*trapz(t,x);

fs_signal = a0/2;

for n = 1:N

an = (1/pi)*trapz(t,x.*cos(n*t));

bn = (1/pi)*trapz(t,x.*sin(n*t));

fs_signal = fs_signal + an*cos(n*t) + bn*sin(n*t);

end

% Plotting

figure;

subplot(2,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Half-Wave Rectified Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(2,1,2);

plot(t,fs_signal,'LineWidth',2);

grid on;

title('Fourier Series Approximation');

xlabel('Time');

ylabel('Amplitude');

Output Waveform:

Result:

The Fourier Series of the half-wave rectified signal was obtained using MATLAB. The

reconstructed waveform using Fourier coefficients closely matched the original half-wave

rectified signal, demonstrating the Fourier Series representation of periodic signals.