Experiment 7.3

Fourier Transform of a Continuous-Time Signal

Aim

Compute and plot Fourier transform magnitude and phase.

Program (MATLAB)

% Fourier Transform of a Continuous-Time Signal

clc;

clear;

close all;

% Time axis

t = -5:0.01:5;

% Continuous-time signal

x = exp(-abs(t));

% Fourier Transform using FFT

X = fftshift(fft(x));

% Frequency axis

N = length(X);

f = linspace(-50,50,N);

% Plotting

figure;

subplot(2,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Continuous-Time Signal x(t)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2,1,2);

plot(f,abs(X),'LineWidth',2);

grid on;

title('Magnitude Spectrum |X(f)|');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

Output Generated Graph

Result

The Fourier Transform of the continuous-time signal x(t)=e−∣t∣ was computed using

MATLAB. The magnitude spectrum was obtained using FFT and showed that most of the

signal energy is concentrated around the zero-frequency component, illustrating the

frequency-domain representation of the signal.