Experiment 4.2

Causal/Non-Causal System

Aim:

To analyze and distinguish between causal and non-causal discrete-time systems. The Aim is to understand how causality affects the system's response to different input sequences.

Program:

% Causal and Non-Causal Systems

clc;

clear;

close all;

% Input sequence

n = 0:10;

x = [1 2 3 4 5 6 7 8 9 10 11];

% Causal System: y(n)=x(n)+x(n-1)

y_causal = filter([1 1],1,x);

% Non-Causal System: y(n)=x(n)+x(n+1)

y_noncausal = x + [x(2:end) 0];

% Plotting

figure;

subplot(3,1,1);

stem(n,x,'filled');

grid on;

title('Input Sequence x(n)');

xlabel('n');

ylabel('Amplitude');

subplot(3,1,2);

stem(n,y_causal,'filled');

grid on;

title('Causal System Output y(n)=x(n)+x(n-1)');

xlabel('n');

ylabel('Amplitude');

subplot(3,1,3);

stem(n,y_noncausal,'filled');

grid on;

title('Non-Causal System Output y(n)=x(n)+x(n+1)');

xlabel('n');

ylabel('Amplitude');

Output Waveform:

Result:

The causal and non-causal discrete-time systems were implemented successfully using MATLAB. The causal system used present and past input samples, whereas the non-causal system used present and future input samples. The corresponding outputs were obtained and displayed using stem plots.