6. CONVOLUTION AND SAMPLING TECHNIQUES

Experiment 6.1

Convolution of Continuous Time Signals

Aim:

To perform the convolution of two continuous-time signals using MATLAB. The Aim is to

understand the concept of convolution as a mathematical operation that combines two signals to produce a third signal, representing the amount of overlap between the two as one signal is shifted over the other.

Program:

% Convolution of Continuous-Time Signals

clc;

clear;

close all

% Time axis

t = 0:0.01:5;

% Input signals

x1 = exp(-t); % Exponential signal

x2 = ones(size(t)); % Unit step signal

% Convolution

y = conv(x1,x2)*0.01;

% Time axis for output

t_conv = 0:0.01:(length(y)-1)*0.01;

% Plotting

figure;

subplot(3,1,1);

plot(t,x1,'LineWidth',2);

grid on;

title('Signal x_1(t)=e^{-t}');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,2);

plot(t,x2,'LineWidth',2);

grid on;

title('Signal x_2(t)=u(t)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,3);

plot(t_conv,y,'LineWidth',2);

grid on;

title('Convolution Output y(t)');

xlabel('Time (s)');

ylabel('Amplitude');

Output Waveform:

Result:

The convolution of the continuous-time signals x1(t)=e−tu(t) and x2(t)=u(t) was performed

successfully using MATLAB. The output signal y(t)=1−e−t was obtained and plotted,

demonstrating the convolution operation for continuous-time systems.