Experiment 6.3

Sampling Theorem Verification

Program (MATLAB)

% Sampling Theorem Verification

clc;

clear;

close all;

% Signal parameters

fm = 5; % Signal frequency (Hz)

fs = 20; % Sampling frequency (Hz)

t = 0:0.001:1; % Continuous time

% Continuous signal

x = sin(2*pi*fm*t);

% Sampled signal

n = 0:1/fs:1;

xs = sin(2*pi*fm*n);

% Plotting

figure;

subplot(2,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Continuous-Time Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2,1,2);

stem(n,xs,'filled');

grid on;

title('Sampled Signal (f_s = 20 Hz)');

xlabel('Time (s)');

ylabel('Amplitude');

Output Generated Graph

Result

The Sampling Theorem was verified using MATLAB by sampling a 5 Hz sinusoidal signal at

a sampling frequency of 20 Hz. Since the sampling frequency was greater than the Nyquist

rate (10 Hz), the signal was sampled without aliasing and could be reconstructed accurately.