Experiment 6.4

Aliasing Effect Demonstration

Aim

Show aliasing when sampling rate is too low.

Program (MATLAB)

% Aliasing Effect Demonstration

clc;

clear;

close all;

% Signal frequency

fm = 10; % Hz

% Sampling frequency (below Nyquist rate)

fs = 15; % Hz

% Continuous-time signal

t = 0:0.001:1;

x = sin(2*pi*fm*t);

% Sampled signal

n = 0:1/fs:1;

xs = sin(2*pi*fm*n);

% Plotting

figure;

subplot(2,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Original Continuous-Time Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2,1,2);

stem(n,xs,'filled');

grid on;

title('Aliased Sampled Signal');

xlabel('Time (s)');

ylabel('Amplitude');

Output Generated Graph

Result.

The aliasing effect was demonstrated successfully using MATLAB. A 10 Hz sinusoidal signal

sampled at 15 Hz (below the Nyquist rate of 20 Hz) appeared as a lower-frequency 5 Hz

signal, confirming the occurrence of aliasing due to undersampling.