Experiment 5.2

Step Response of the System

Expected Learning Outcomes

After completion of this experiment, students will be able to:

1. Define and generate a step input signal to study system behavior under sudden input

changes.

2. Model a discrete-time system using numerator and denominator coefficients of a

difference equation.

3. Compute the step response of a system using MATLAB’s filter() function.

4. Analyze and interpret the input and output response plots to understand system

characteristics and behavior.

Aim:

To analyze the step response of a discrete-time system. The step response characterizes how

the system responds to a step input, which is a common way to assess the transient and

steady-state behavior of the system.

System Definition:

Program:

% Step Response of the System

clc;

clear;

close all;

% Sample index

n = 0:10;

% Unit step input

x = ones(1,length(n));

% System: y(n)=x(n)+0.5x(n-1)

b = [1 0.5];

a = [1];

% Step response

y = filter(b,a,x);

% Display result

disp('Step Response y(n):');

disp(y);

% Plot

figure;

subplot(2,1,1);

stem(n,x,'filled');

grid on;

title('Unit Step Input u(n)');

xlabel('n');

ylabel('Amplitude');

subplot(2,1,2);

stem(n,y,'filled');

grid on;

title('Step Response of the System');

xlabel('n');

ylabel('y(n)');

Output Waveform:

Result:

The step response of the discrete-time system y(n)=x(n)+0.5x(n−1) was obtained by

applying a unit step input. The output reached a steady-state value of 1.5, demonstrating the

response of the system to a constant input signal.