5. System Response and LTI Properties

Experiment 5.1

Impulse Response of the System

Aim:

To determine and plot the impulse response of a continuous-time system. This involves

applying an impulse function δ(t)\delta(t)δ(t) as the input and observing the system's output.

Program:

% Impulse Response of the System

clc;

clear;

close all;

% Sample index

n = 0:10;

% Unit impulse input

x = [1 zeros(1,10)];

% System: y(n)=x(n)+0.5x(n-1)

b = [1 0.5];

a = [1];

% Impulse response

h = filter(b,a,x);

% Display result

disp('Impulse Response h(n):');

disp(h);

% Plot

figure;

stem(n,h,'filled');

grid on;

title('Impulse Response of the System');

xlabel('n');

ylabel('h(n)');

Output Waveform:

Result:

The impulse response of the discrete-time system y(n)=x(n)+0.5x(n−1) was obtained by

applying a unit impulse signal as input. The resulting impulse response was h(n)={1,0.5,0,0,...}, which characterizes the behavior of the system.